Update SOP for Serial to TCP/IP Converter

- Please connect all the devices (i.e. COM port, HUB, Serial to TCP/IP, Power Adapter) together as shown below (Please be sure to use cross type RS-232 cable).

- Please save the firmware image file and update utility file (as below example files) to a certain path of your PC, and remember its path.

(1) sflash.exe

(2) 100529-LM6911-S21-V1.15.bin

- Please go to “DOS” command.

- Please switch to the path where you save the firmware image file and update utility file (i.e. E:\ as shown below).

Please DO NOT execute update yet, because the Serial to TCP/IP converter must be set to “Firmware update mode” first as next step.

- Please execute “CVBrowser.exe” and choose “Device Name".

- Please click on “update firmware” as below screenshot:

- After click on “confirm”, the SYS LED lamp of the Serial to TCP/IP converter should blink for seconds then constantly light up (which means the Serial to TCP/IP converter is already in updating mode).

- From DOS mode, go back to the path where you save the firmware image file and update utility file.

- Please type “Sflash xxxxxxxxx.bin -p 0x1000 -c1 –d” as shown below:

(xxxxxxxxx.bin means the filename of the firmware image file)

- Then firmware updating is in processing.  When processing is done, it will show “Successfully downloaded” to the device.” As below screenshot.